Computer Architecture

Fall, 2022

Week 3

2021.09.26

組別：＿＿＿＿＿　簽名：＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿

[group1]

For the following two C statements, what are the corresponding MIPS assembly codes? Assume that the variables f, g, h, i, and j are assigned to registers $s0, $s1, $s2, $s3, and $s4, respectively.

- f = g – h;

- f = h + (g - 3);

j = f;

[group7]

- Why registers are easier for compiler to use?

- Why is the memory address represented by pointer and offset in the data transfer instructions?

- What are the definition of “Spilling” and “Alignment”?

[group4]

Based on MIPS instruction set architecture, which of the following statements are true?

- There are 32 general purpose registers and 32 floating point registers.

- If we want to access sequential data in memory by word address, 872 and 954 would be accessed.

- If our program frequently uses a certain constant x, we can save x into the $zero register to speed up the whole program.

- The desired memory address is the sum of an offset (in byte) and a register containing a pointer to memory.

[group12]

In this week’s online video, we have learned three design principles. Please answer principle’s names and give an example for each one.

[group10]

True or False and explain if it is false.

- There are 32 MIPS registers and each is 32 bits wide.

- The content in $s1 is 6, and the content in $0 after the following instruction will be 10.

addi $0,$s1,4

- To implement the following code($s1 : b, $s2 : c, $s3 : base on address of A) in C language:

c = b + A[4]

First, we need to transfer memory to register first by

lw	$t0, 4($s3)		#$t0 receives A[4]

Next, add it to b and replace c

add	$s1, $s2, $t0

- Registers stored in the processor can improve code density since compared to memory location, registers are named with fewer bits.

- We can keep hardware simple via regularity, e.g. keeping syntax rigid. The benefit is that the implementation will be simpler.

- lw(load word) means to transfer word from register to memory; sw(store word) means to transfer word from memory to register.

- Assembly language uses variables to operate just like high-level language.

[group13]

Please select the correct options.

- It’s more difficult to access memory than register. We use base register and offset to access memory efficiently.

使用memory比register 更困難，因此我們用base register和offset 會使memory更有效率

- If assembly operands have been in memory, it's NOT necessary to load them to registers.

不需要load assembly operands 到register

- Even if we define a fixed and finite number of variables in high level language, the compiler may NOT use the same number of variable registers.

即使我們在hign level language中定義了固定且有限數量的變數，但是在compiler中可能不會使用相同數量的變數在 registers。

- Memory alignment helps access memory more efficiently in most case.

Memory alignment 幫助我們更有效率的使用memory

- To reduce the usage of hardware resources, MIPS instructions may have different length.

為了減少hardware resources，MIPS instruction 會有不同的長度

- lw, sw, ..., are data transfer instructions. When we talk about load and store instructions, the direction of data flow is viewed from the aspect of register.

Lw,sw 是data轉換的instruction 我們所說的load和store 是根據register的角度來看

- If registers are NOT full used, the least used variable can still stay in registers.

如果register沒有全部使用，最常使用的variable 會留在register

[group14]

Assume $s0 = 1000. Given the memory table M and following instructions, what is the value of  M[1016]?

lw	$t0,	0($s0)

lw	$t1,	4($s0)

add	$t2,	$t0,	$t1

sw	$t2,	16($s0)