Computer Architecture

Fall, 2022

Week 8

2021.10.30

組別：＿＿＿＿＿　簽名：＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿＿

Group6

Which of the following statements are true?

(A) 0 10000101 01001100000000000000000 in IEEE 754 represents 83.375 in decimal.

(B) 1 10000010 10010100000000000000000 in IEEE 754 represents -12.625 in decimal.

(C)If some values are divided by zero, MIPS will raise an exception.

(D)In bias 15, 01101 represents -2.

Group14

Please explain/fill in the following according to the IEEE 754 standard:

- Why is there a need for the designation of denormal (subnormal numbers) in the standard?

- Why does the value of the mantissa (significand) always ignores the digit left to the decimal point?

- Why is there a need for the biased notation (instead of a 2’s complement representation for signed numbers)?

- How many different NaN values are there in the single-precision floating point number standard? (Answer can be presented in exponent notation)

Group 2

Please fill out the table according to steps of 1011/0110 and the following flow chart. Write down Quotient and Remainder.

| Step | Remainder |  | Divisor | Description |
| --- | --- | --- | --- | --- |
| 0 | 0000 | 1011 | 0110 | Initialization |

Quotient: 	Remainder:

Hint:

To fill the Description, there are some options:

- Shift xxxxxxxxx left/right

- xxxxxxxxx < 0 / > 0

- Restore original value

- Subtract/Add xxxxxxx

- Set the new significant bit to 1/0

……

Group4

Which of the following statements are true?

- The unsigned multiplier of two 32-bit numbers requires a 32-bit register for multiplicand and a 32-bit register for product.

- Based on 32-bit IEEE 754 standard’s single precision, no other floating point number is greater than 0x7f800000.

- Hi and Lo registers are used in both multiplication and division, and Hi would store the quotient in division.

- If there were only 16 bits for significand field in floating point representation, it is equivalent to 4 decimal digits of precision.

- For 32-bit unsigned division, we only need 32 iterations and shift one register to get the correct result.

- By IEEE-754 single precision floating-point representation, the largest positive normalized number is +(1-2-23)×2+127.

- Exponents with all 1’s are reserved for ±∞ and NaN.

Group12

True or False:

- when we use mult $t1, $t2, we will push most significant 32 bits to lo and least significant  32 bits to hi.

- In multiply version 2 we will place multiplier to product register’s right hand and shift right until the multiply end.

- Divide version 1 and multiply version 1 have same repetition times.

- when we use div $t1, $t2, we will push remainder to hi and quotient to lo, and we can use mflo $t3 and mfhi $t4 to copy the lo and hi value to register t3 and t4.

- For 32-bit IEEE 754 floating-point standard, the smallest positive single precision denormalized number is: 0.0000 0000 0000 0000 0000 0012 x 2 ^ -126.

- 0.687510 = 0.01112

- In the IEEE 754 floating-point representation, the precision of represented numbers is determined by the size of exponent.

- In the IEEE 754, we use 2’s complement in exponent field .

Group1

Below are some steps for performing a basic floating-point multiplication. Please order the steps.

- Normalize the product and check for overflow/underflow when shifting

- Add the exponents of operands to get the exponent of the product

- Round the mantissa and renormalize when necessary

- Multiply the mantissa of operands

- Set the sign of the product

Group 7

Half-precision floating-point (FP16) has 1 bit of signed bit, 5 bits of exponent, and 10 bits of mantissa. The exponent uses bias of 15.

For the following question, calculate the results and represent them in FP16 bit representation:

1) 13(10)

2) 1.111(2) * 2^-14 - 1.000(2) * 2^-13

3) 1024(10) * 512(10)

(hint : 512 = 2^9, 1024 = 2^10)